To distinguish left from right there is a gap between the C and D bands.

- band A is first significant figure of component value (left side)

- band B is the second significant figure

- band C is the decimal multiplier

- band D if present, indicates tolerance of value in percent (no color means 20%)

The standard color code per EN 60062:2005 is as follows:

| H | F | Ohm | Color | Significant figures | Multiplier | Tolerance |  | Temp. Coefficient (ppm/K) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10µ's | 10p's | 10's | Black | 0 | ×100 | – |  | 250 | U |
| 100µ's | 100p's | 100's | Brown | 1 | ×101 | ±1% | F | 100 | S |
| 1m's | 1n's | 1k's | Red | 2 | ×102 | ±2% | G | 50 | R |
| 10m's | 10n's | 10k's | Orange | 3 | ×103 | – |  | 15 | P |
| 100m's | 100n's | 100k's | Yellow | 4 | ×104 | – |  | 25 | Q |
| 1's | 1µ's | 1M's | Green | 5 | ×105 | ±0.5% | D | 20 | Z |
| 10's | 10µ's | 10M's | Blue | 6 | ×106 | ±0.25% | C | 10 | Z |
| 100's | 100µ's | 100M's | Violet | 7 | ×107 | ±0.1% | B | 5 | M |
| 1k's | 1m's | 1G's | Gray | 8 | ×108 | ±0.05% | A | 1 | K |
| 10k's | 10m's | 10G's | White | 9 | ×109 | – |  | – |  |
| 1µ's | 1p's | 1's | Gold | – | ×10-1 | ±5% | J | – |  |
| 1/10µ's | 1/10p's | 1/10's | Silver | – | ×10-2 | ±10% | K | – |  |
| - | - | - | None | – | – | ±20% | M | – |  |
| - Any temperature coefficent not assigned its own letter shall be marked "Z", and the coefficient found in other documentation. - For more information, see EN 60062. |  |  |  |  |  |  |  |  |  |  |