Refer to PIcaxe manual 2 and check up the following commands:

Readadc

If … then

Let

Goto

For … next

Pwmout

Pause

debug

Use the above commands (or even more) and write a programme to perform:

- Ramp up and down an LED

- Ramp up and down three LEDs of different colours

- Ramp up and down an LED at different rates according to the ambient level of light (you may need to introduce a variable and go from there)

What is shown below may be part of your programme:

for b0=0 to 255 step 10

pwmout b.2, 64, b0

debug b0

next b0

pause 100

Do your best and see how far you can go within a session.

When simulating your programme, those with Mac may need to join those with Window laptops as the MacAXEpad does not have a simulator.